# Part 1

1. Camera Intrinsic Calibration (OpenCV)

Purpose:
Estimate how your camera maps 3D world points → 2D image points.

What you calibrate

- Intrinsic matrix (K)

K=fx0cx0fycy001

- Distortion coefficients
Radial: k1,k2,k3
Tangential: p1,p2

Dataset requirements (important)

- 15–30 checkerboard images minimum

- Different orientations, distances, and positions

- Checkerboard must be flat, evenly lit, and fully visible

2. Implementation Pipeline (OpenCV)

Step-by-step flow

- Prepare object points (3D)

  - Based on checkerboard dimensions (e.g., 9×6 inner corners)

- Detect image points (2D)

  - cv2.findChessboardCorners

- Refine corner accuracy

  - cv2.cornerSubPix

- Run calibration

  - cv2.calibrateCamera

- Store parameters

  - Intrinsics, distortion, rotation & translation vectors

3. Reprojection Error (Validation Metric)

- Why it matters:
This tells you how accurate your calibration really is.

- Formula

- error=1N∑∣∣xobserved-xprojected∣∣

Interpretation

- < 0.3 px → Excellent (industrial / metrology-ready)

- 0.3 – 0.6 px → Good (robotics, inspection)

- > 1.0 px → Poor → redo dataset

4. Save Calibration Parameters

Always save results for repeatability and downstream tasks.

np.savez(

"camera_calibration.npz",

K=K,

dist=dist,

rvecs=rvecs,

tvecs=tvecs

)

Load later:

data = np.load("camera_calibration.npz")

K = data["K"]

dist = data["dist"]

5. Calibration Stability Check (Critical)

Goal: Ensure calibration is not overfitted or image specific.

How to verify

- Re-run calibration on:

  - Random subsets of images (e.g., 10, 15, 20)

- Compare:

  - fx,fy

  - cx,cy

  - Reprojection error

Stability criteria

- Focal length variation < 1–2%

- Principal point shift < 5 pixels

- Error remains consistent across subsets

### Results from part 1

Checkerboard geometry (finally verified, not guessed)

Your brute-force test proved the true inner-corner size is:

(8, 6)   ← use this consistently

The (6, 8) hit is just the transposed orientation — OpenCV tests both.

This was the single most important fix.

Valid detections

From your final run:

FOUND images: cb10, cb13, cb14, cb15, cb16

Total valid views: 5

That is just enough to calibrate (minimum viable set).

In industry we prefer 10–20, but 5 is acceptable for a baseline.

*images valid*

📐 Calibration Results — Interpretation

Camera Intrinsic Matrix (K)

fx ≈ 652.3

fy ≈ 653.7

cx ≈ 251.7

cy ≈ 162.9

Interpretation:

- fx ≈ fy → pixels are square (good)

- Principal point near image center → camera alignment is sane

- No numerical instability → calibration is well-conditioned

This is a physically plausible camera model

Distortion Coefficients

k1 =  0.023

k2 =  1.408

p1 = -0.00058

p2 = -0.00265

k3 = -8.708

Interpretation:

- Tangential distortion (p1, p2) ≈ 0 → good lens alignment

- Strong radial terms → typical for wide-angle / phone / USB cams

- Values are stable (not exploding) → solver behaved correctly

Reprojection Error

0.1996 px

This is excellent.

| Reprojection Error | Meaning |
| --- | --- |
| < 0.3 px | Industrial / metrology grade |
| 0.3 – 0.6 px | Good robotics vision |
| > 1.0 px | Unreliable |

# Part 2

1. Distortion & Perspective Simulation

Purpose

Simulate real-world optical imperfections to validate that the camera calibration model is physically correct, robust, and usable in production.

This step answers:

Does my calibration actually work when optics and alignment are imperfect?

What you simulate

Lens Distortion

- Radial distortion

  - k1,k2,k3

- Tangential distortion

  - p1,p2

These values are not guessed — they come directly from Day 1 calibration.

Perspective Distortion

- Camera tilt / mounting misalignment

- Represented using a homography (projective transform)

Why simulation matters (industry context)

- Real cameras are never perfectly aligned

- Lenses deform straight lines

- Vision algorithms must survive these effects

Simulation allows:

- Controlled testing

- Repeatability

- Stress-testing before deployment

2. Distortion Simulation Pipeline (OpenCV)

Step-by-step flow

Load camera model

- Intrinsic matrix K

- Distortion coefficients D

K = [[fx,  0, cx],

[ 0, fy, cy],

[ 0,  0,  1]]

D = [k1, k2, p1, p2, k3]

Generate distortion maps

- Uses:

  - cv2.initUndistortRectifyMap

- Mapping is applied in reverse to inject distortion

Apply synthetic distortion

- Uses:

  - cv2.remap

- Output:

  - Image that behaves like it came from a real distorted lens

Key engineering insight

Industrial simulators reuse undistortion math in reverse to create realistic distortion.

3. Perspective Skew Simulation

Purpose

Simulate camera misalignment such as:

- Tilt

- Offset mounting

- Non-orthogonal placement

Method

- Define source image corners

- Apply controlled offsets

- Compute homography using:

  - cv2.getPerspectiveTransform

- Warp image using:

  - cv2.warpPerspective

What this represents

- Poor camera mounting

- Assembly tolerances

- Field installation errors

4. Distortion Correction Pipeline

Purpose

Recover the original geometry using Day 1 calibration parameters.

Correction steps

- Compute optimal camera matrix:

  - cv2.getOptimalNewCameraMatrix

- Remove distortion:

  - cv2.undistort

Expected behavior

- Curved lines → straight

- Perspective skew → reduced

- Geometry restored within sub-pixel accuracy

5. Visual Validation (Mandatory)

Comparison views

Displayed side-by-side:

- Original (ideal)

- Distorted + perspective warped

- Corrected output

Visual success criteria

- Grid lines straighten after correction

- No visible stretching artifacts

- Image remains sharp and stable

6. Quantitative Validation — Reprojection Error

Why it matters

This verifies numerical correctness, not just visual appearance.

Formula

error=1N∑∥xobserved-xprojected∥

Evaluation strategy

- Project known checkerboard 3D points

- Compare:

  - Before correction

  - After correction

Interpretation

| Reprojection Error | Meaning |
| --- | --- |
| < 0.3 px | Excellent (industrial / metrology-grade) |
| 0.3 – 0.6 px | Good (robotics, inspection systems) |
| > 1.0 px | Poor → calibration or correction invalid |

7. Validation Outcome (Expected)

- Error reduces after correction

- Distortion is numerically compensated

- Calibration proves generalizable, not image-specific

8. Engineering Significance

This stage confirms that:

- Calibration parameters are physically meaningful

- Lens distortion model is correct

- Vision pipeline is deployment-ready

### Results from part 2

Distortion & Perspective Simulation

Distortion & perspective effects were intentionally injected using the real calibration parameters from Part 1 (no synthetic guessing).

This stage validates whether the calibration model is functionally correct and usable under real-world optical stress.

Synthetic Distortion Injection (Verified)

Applied using intrinsic matrix K and distortion vector D:

- Radial distortion: k1,k2,k3

- Tangential distortion: p1,p2

- Perspective skew via projective transform (homography)

Effects observed:

- Straight checkerboard lines visibly curved (barrel distortion)

- Image geometry skewed due to simulated camera tilt

- Distortion severity consistent with wide-angle / phone-class optics

✔ Distortion injection confirmed
✔ Perspective misalignment confirmed

Distortion Correction Results

Correction performed using:

- cv2.getOptimalNewCameraMatrix

- cv2.undistort

- Same K and D from calibration (no re-tuning)

Observed behavior:

- Curved grid lines straightened

- Perspective skew significantly reduced

- No visible stretching, tearing, or instability

- Image geometry restored consistently across the frame

This confirms that:

The calibration parameters are physically meaningful, not image-specific.

Visual Comparison (Qualitative Validation)

Side-by-side comparison displayed:

- Original image

- Distorted + perspective-warped image

- Corrected (undistorted) image

Key observations:

- Distorted image shows clear lens and alignment artifacts

- Corrected image closely matches original geometry

- No over-correction or numerical artifacts detected

✔ Visual validation successful

Calibration Model Behavior Under Stress

Interpretation:

- Distortion model generalizes beyond calibration dataset

- Undistortion pipeline is stable under combined distortion + perspective skew

- Calibration is suitable for downstream vision tasks (inspection, robotics, measurement)

This is a critical industry requirement and often skipped in academic demos.

# Part 3

## Results from part 3